Chemical Industries

Name: _______________________________

Cell number: ________________________

Email address: ________________________

Fertilizers

| Definitions: |  |
| --- | --- |
| Eutrophication | Is the process wherein the ecosystem e.g. a river, is enriched with inorganic plant nutrients, especially phosphorus and nitrogen that leads to excessive vegetation. |

Nutrients:

| Macro nutrients | Non-mineral | Necessary | C, H, O |
| --- | --- | --- | --- |
|  | Mineral | Primary nutrients | N, P, K |
|  | Mineral | Secondary nutrients | Ca, Mg, S |
| Micro nutrients | Mineral |  | B, Cu, Fe, Mn, Zn |

C, H and O are obtained from the atmosphere (CO2) and rain (H2O).

Importance of Fertilizers:

•	Farmers have to produce more food yearly in order to feed the growing world population.

•	Nutrient shortages arise in the ground and decreases crops.

•	If nutrients are not supplemented, then even indigenous plants are endangered.

•	Without today’s commercially available fertilizers it would not have been possible to maintain the human population.

| Nutrient | Source | Function in plants |
| --- | --- | --- |
| Nitrogen (N) | - From the air - Ghwano | - Needed for the forming of protein and enzymes. - Improves leaf growth. |
| Phosphorus (P) | - Bone meal - Superphosphate | - Development of roots, stems and leaves. |
| Potassium (K) | - Organic plants and animal materials | - Resistance against illness - Protects the stem |

Preparation of Hydrogen:

- In SA hydrogen is obtained from SASOL.

- Methane gas is oxidised (burnt) to form CO and H2.

2CH4 + O2  2CO + 4H2

- SASOL also utilizes the steam reforming of coal to produce hydrogen and this is known as gasification.

C(g) + H2O(l)  CO(g) + H2(g)

- Together, the carbon monoxide and hydrogen is a synthetic gas, also known as singas.

Industrial preparation of Ammonia – The Haber process

Ammonia is produced industrially according to the Haber process.  Atmospheric nitrogen and hydrogen are compressed in a special container at pressures of 200 – 1000 atmospheres and heated at a temperature of plus/minus 500oC.  In the presence of a suitable catalyst, like FeO, N2(g) and H2(g) bind to form ammonia. The ammonia is removed by liquefying it or by dissolving it in water.

Reaction equation: 	N2(g) + 3H2(g)   2NH3(g) …………Fe of FeO (as catalyst)

Industrial preparation – The Ostwald process

Nitric acid is prepared at a large scale in the industry through the Ostwald process.

The process can be divided into the following steps:

- Forming of the first oxide.

Pure ammonia burns in oxygen in the presence of platinum gauze (as a contact catalyst) and forms NO(g).  This step is known as the catalytic oxidation of ammonia.

4NH3(g) + 5O2(g) → 4NO(g) + 6H2O            4-5-4-6

Pt

- The oxide is oxidized further.

The NO(g) formed reacts with excess oxygen and forms NO2(g).

2NO(g) + O2(g) → 2NO2(g)

- Adding of water.

NO2(g) is gurgled through water and nitric acid and more NO forms. The NO that forms is sent back to the second step of the process.

3NO2(g) + H2O → 2HNO3 + NO(g)              3-1-2-1

Industrial preparation – The Contact process

H2SO4 is prepared in the Contact process.

The following steps represent the process:

- Forming of the first oxide.

Sulphur, or a sulphur sulphide like FeS, is burnt to form SO2(g):

S + O2(g)   SO2(g)

- The oxide is oxidised further.

In the reversed-, or contact jar, SO2(g) combines with O2(g) in the presence of a catalyst (V2O5 or platinum) to form SO3:

Pt

2SO2(g) + O2(g)           2SO3(g)

V2O5

- Adding of water.

If H2O is added to SO3 , Sulfuric acid is formed, but it forms a fine mist that can create practical problems.  The adding of water is therefore done in two steps: The SO3(g) comes in contact with the concentrated sulphuric acid to form spiro sulphuric acid (smoking sulphuric acid or oleum):

SO3 + H2SO4     H2S2O7

The spiro sulphuric acid is then diluted with water to form sulphuric acid:

H2S2O7 + H2O   2H2SO4

Ammonium sulphate

Ammonium nitrate

Nitric acid

Ostwald Process

Sulphuric acid

Contact process

Ammonia

Haber Process

Hydrogen

Nitrogen

Fractional Distillation

N2

H2

(NH4)2SO4

NH4NO3

NH3

HNO3

H2SO4